Задание 1

CHIP Bit {

IN in, load;

OUT out;

PARTS:

Mux(a=outDFF, b=in, sel=load, out=outMux);

DFF(in=outMux, out=outDFF, out=out);

}

Задание 2

CHIP Register {

IN in[16], load;

OUT out[16];

PARTS:

Bit(in=in[0], load=load, out=out[0]);

Bit(in=in[1], load=load, out=out[1]);

Bit(in=in[2], load=load, out=out[2]);

Bit(in=in[3], load=load, out=out[3]);

Bit(in=in[4], load=load, out=out[4]);

Bit(in=in[5], load=load, out=out[5]);

Bit(in=in[6], load=load, out=out[6]);

Bit(in=in[7], load=load, out=out[7]);

Bit(in=in[8], load=load, out=out[8]);

Bit(in=in[9], load=load, out=out[9]);

Bit(in=in[10], load=load, out=out[10]);

Bit(in=in[11], load=load, out=out[11]);

Bit(in=in[12], load=load, out=out[12]);

Bit(in=in[13], load=load, out=out[13]);

Bit(in=in[14], load=load, out=out[14]);

Bit(in=in[15], load=load, out=out[15]);

}

Задание 3

CHIP RAM8 {

IN in[16], load, address[3];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address, a=load1, b=load2, c=load3, d=load4, e=load5, f=load6, g=load7, h=load8);

Register(in=in, load=load1, out=out1);

Register(in=in, load=load2, out=out2);

Register(in=in, load=load3, out=out3);

Register(in=in, load=load4, out=out4);

Register(in=in, load=load5, out=out5);

Register(in=in, load=load6, out=out6);

Register(in=in, load=load7, out=out7);

Register(in=in, load=load8, out=out8);

Mux8Way16(a=out1, b=out2, c=out3, d=out4, e=out5, f=out6, g=out7, h=out8, sel=address, out=out);

}

Задание 4

CHIP RAM64 {

IN in[16], load, address[6];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[3..5],

a=load1, b=load2, c=load3, d=load4,

e=load5, f=load6, g=load7, h=load8);

RAM8(in=in, load=load1, address=address[0..2], out=out1);

RAM8(in=in, load=load2, address=address[0..2], out=out2);

RAM8(in=in, load=load3, address=address[0..2], out=out3);

RAM8(in=in, load=load4, address=address[0..2], out=out4);

RAM8(in=in, load=load5, address=address[0..2], out=out5);

RAM8(in=in, load=load6, address=address[0..2], out=out6);

RAM8(in=in, load=load7, address=address[0..2], out=out7);

RAM8(in=in, load=load8, address=address[0..2], out=out8);

Mux8Way16(a=out1, b=out2, c=out3, d=out4,

e=out5, f=out6, g=out7, h=out8,

sel=address[3..5], out=out);

}

Задание 5

CHIP RAM512 {

IN in[16], load, address[9];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[6..8],

a=load1, b=load2, c=load3, d=load4,

e=load5, f=load6, g=load7, h=load8);

RAM64(in=in, load=load1, address=address[0..5], out=out1);

RAM64(in=in, load=load2, address=address[0..5], out=out2);

RAM64(in=in, load=load3, address=address[0..5], out=out3);

RAM64(in=in, load=load4, address=address[0..5], out=out4);

RAM64(in=in, load=load5, address=address[0..5], out=out5);

RAM64(in=in, load=load6, address=address[0..5], out=out6);

RAM64(in=in, load=load7, address=address[0..5], out=out7);

RAM64(in=in, load=load8, address=address[0..5], out=out8);

Mux8Way16(a=out1, b=out2, c=out3, d=out4,

e=out5, f=out6, g=out7, h=out8,

sel=address[6..8], out=out);

}

CHIP RAM4K {

IN in[16], load, address[12];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[9..11],

a=load1, b=load2, c=load3, d=load4,

e=load5, f=load6, g=load7, h=load8);

RAM512(in=in, load=load1, address=address[0..8], out=out1);

RAM512(in=in, load=load2, address=address[0..8], out=out2);

RAM512(in=in, load=load3, address=address[0..8], out=out3);

RAM512(in=in, load=load4, address=address[0..8], out=out4);

RAM512(in=in, load=load5, address=address[0..8], out=out5);

RAM512(in=in, load=load6, address=address[0..8], out=out6);

RAM512(in=in, load=load7, address=address[0..8], out=out7);

RAM512(in=in, load=load8, address=address[0..8], out=out8);

Mux8Way16(a=out1, b=out2, c=out3, d=out4,

e=out5, f=out6, g=out7, h=out8,

sel=address[9..11], out=out);

}

CHIP RAM16K {

IN in[16], load, address[14];

OUT out[16];

PARTS:

DMux4Way(in=load, sel=address[12..13], a=load1, b=load2, c=load3, d=load4);

RAM4K(in=in, load=load1, address=address[0..11], out=out1);

RAM4K(in=in, load=load2, address=address[0..11], out=out2);

RAM4K(in=in, load=load3, address=address[0..11], out=out3);

RAM4K(in=in, load=load4, address=address[0..11], out=out4);

Mux4Way16(a=out1, b=out2, c=out3, d=out4, sel=address[12..13], out=out);

}

Задание 6